Instruction manual

Small bee Smart trigeminal pattern + bee's eye

Thank you for choosing our trigeminal pattern + bee's eye,For your personal safety and better use of this product,Please read the manual carefully before using,Follow the procedure,To avoid misuse of personal safety damage and lamp damage.

- Packing list

- After unpacking,Please check the product is qualified,Count all accessories.NO, please contact the distributor!

| - number | - Name | - Color | - Unit | - Quantity |
| --- | --- | --- | --- | --- |
| - 1 | - Lamps and lanterns | - Black | - Station | - 1 |
| - 2 | - Power Cord | - Black | - Ordinance | - 1 |
| - 3 | - Handle | - Black | - PCS | - 1 |
| - 4 | - Hexagon screws | - Black | - PCS | - 2 |
| - 5 | - Instructions | - Black and white | - PCS | - 1 |

- Appearance and LED Function

- Switch settings by pressing a button：

- *Click“MENU”Key switch function.

- *Click“UP/DOWN”Switch function or set address code.

- *After switching functions，Click“ENTER”Key save function.

- 三、Technical parameters

| - number | - Category | - Parameter |
| --- | --- | --- |
| - 1 | - Power supply | - AC95V-245V/50Hz |
| - 2 | - Product power | - 150W |
| - 3 | - Pattern parameters | - 8 PCS patterns, 8 PCS colors |
| - 4 | - Lamp beads | - 1 80W white light lamp bead, 6 5050 10W RGBW lamp beads |
| - 5 | - Control | - DMX512, 15 channels for international use |
| - 6 | - Mode of operation | - DMX512, master/slave, self-walking, voice control |
| - 7 | - Dimming mode | - 0~100% ultra-smooth dimming |
| - 8 | - Strobe | - 20 times per second |
| - 9 | - Heat dissipation | - Aluminum products cooling, forced cooling fan |

- 四、DMX Channel function:

- This product have 15 channels，Specific controls are as follows:

| - channels | - Function | - DMX Value | - Description |
| --- | --- | --- | --- |
| - 1 | - X-axis motor | - 0-255 | - 0~255 0-540 degree rotation |
| - 2 | - Y-axis motor | - 0-255 | - 0~255 0-180 degree rotation |
| - 3 | - XY axis motor speed | - 0-255 | - 0~255 speed from fast to slow |
| - 4 | - dimming | - 0-255 | - Linear dimming from dark to light |
| - 5 | - Stroboscopic | - 0-9 | - None |
|  |  | - 10-255 | - Slow to fast |
| - 6 | - colour disk | - 0-255 | - 0-35 (switch one color every 5 intervals), 35-127 (the last color), 127-255 (color moves from fast to slow) |
| - 7 | - Pattern plate | - 0-255 | - 0-35 (switch one pattern every 5 intervals), 35-127 (the last pattern), 127-255 (the pattern moves from fast to slow) |
| - 8 | - Z turntable | - 0-255 | - 0-127 (0-360 degrees), 128-191 (speed from slow to fast) 192-255 speed from fast to slow |
| - 9 | - Bee eye total dimming | - 0-255 | - Linear dimming from dark to light |
| - 10 | - Bee-eye strobe | - 0-9 | - No function |
|  |  | - 10-255 | - Slow to fast |
| - 11 | - red | - 0-255 | - Linear dimming from dark to light |
| - 12 | - green | - 0-255 | - Linear dimming from dark to light |
| - 13 | - blue | - 0-255 | - Linear dimming from dark to light |
| - 14 | - white | - 0-255 | - Linear dimming from dark to light |
| - 15 | - Macro function | - 0-50 | - Channels 1 to 14 are valid |
|  |  | - 51-150 | - autorun |
|  |  | - 150-250 | - voice |
|  |  | - 251-255 | - 3 second reset |

- menu

| - addr | - A001-A512 | - Set 512 address |
| --- | --- | --- |
| - Chnd | - 15ch | - Display the number of channels |
| - Shou(running mode) | - d512 | - DMX512 mode |
|  | - auto | - Self-propelled mode |
|  | - sond | - Audio Mode |
| - Sens | - 0-100 | - Voice control zero density setting |
| - LED | - oFF | - Display is always on |
|  | - on | - how Hibernate |
| - Disp | - OFF | - Show Forward |
|  | - ON | - Show Reverse |
| - rst | - Yes/no | - Manual reset |

五、Maintenance

- Do not stare at the luminescent body for a long time，It hurts the eyes;

- Clean regularly Lens Keep the brightness and Extension Lamps and lanterns Service life.