 48RGB+8W Stroboscope Lamp 

Technical parameter
  Voltage: AC100-240V 50/60HZ

Power:140W
  Light source: 768pcs 5050RGB 3in1 LED + 96pcs White  LED

  Channel:4/6/12/144/154CH
Operation: DMX512, Auto, Sound, Master-slave

The DMX channel menu

12 Channel (basic function)
	channel
	value
	function declaration

	CH 1
	000-255
	Total dimming, from dark to bright

	CH 2
	005-255
	RGB strobe, from slow to fast (W strobe is CH6)

	CH 3
	000-005
	Perform the CH9-CH11, RGB dimming

	
	006-010
	Static color, CH4 Select (red, green, blue, yellow, purple, cyan, white)

	
	011-015
	Breathing effect, CH5 Select color

	
	016-020
	Jump change effect

	
	021-025
	Gradient change effect

	
	026-030
	Pulse change effect, CH5 Select color

	
	031-032
	Autorun effect-1, CH5 Select color (cycle autorun)

	
	033-034
	Autorun effect-2, CH5 Select color

	
	...
	...

	
	...
	...

	
	203-204
	Autorun effect-87, CH5 Select color

	
	205-229
	Autorun effect-88 (full color effects)

	
	230-233
	Sound mode 1, CH5 Select color

	
	234-237
	Sound mode 2, CH5 Select color

	
	...
	...

	
	246-249
	Sound mode 5, CH5 Select color

	
	250-255
	Sound mode 6, CH5 Select color

	CH 4
	000-255
	Autorun effect: adjustment speed, from slow to fast .
Sound mode: adjustment sensitivity, from low to high (The highest sensitivity is the sensitivity set by the menu) .

	CH 5
	000-031
	Default color

	
	032-063
	Red, CH3 Manual color selection

	
	064-095
	Green, CH3 Manual color selection

	
	096-127
	Blue, CH3 Manual color selection

	
	128-159
	Yellow, CH3 Manual color selection

	
	160-191
	Purple, CH3 Manual color selection

	
	192-223
	Cyan, CH3 Manual color selection

	
	224-255
	White, CH3 Manual color selection

	CH 6
	005-255
	W strobe, from slow to fast (RGB strobe is CH2)

	CH7
	000-005
	Perform the CH12, W dimming

	
	006-010
	W effect-1 (cycle effect)

	
	011-015
	W effect-2 (W static effect)

	
	...
	...

	
	...
	...

	
	206-210
	W effect-41

	
	211-255
	W effect-42

	CH 8
	000-255
	W effect adjustment speed, from slow to fast

	CH 9
	000-255
	R red dimming, from dark to bright

	CH 10
	000-255
	G Green dimming, from dark to bright

	CH 11 
	000-255
	B Blue dimming, from dark to bright

	CH 12
	000-255
	W White dimming, from dark to bright


4 Channel (whole lamp)
	channel
	value
	function declaration

	CH 1
	000-255
	R red dimming, from dark to bright

	CH 2
	000-255
	G Green dimming, from dark to bright

	CH 3
	000-255
	B Blue dimming, from dark to bright

	CH 4
	000-255
	W White dimming, from dark to bright


6 Channel (whole lamp)
	channel
	value
	function declaration

	CH 1
	000-255
	Total dimming, from dark to bright

	CH 2
	000-255
	Total strobe, from slow to fast

	CH 3
	000-255
	R red dimming, from dark to bright

	CH 4
	000-255
	G Green dimming, from dark to bright

	CH 5
	000-255
	B Blue dimming, from dark to bright

	CH 6
	000-255
	W White dimming, from dark to bright


144 Channel (point control)
	channel
	value
	function declaration

	CH 1
	000-255
	R1 red dimming, from dark to bright

	CH 2
	000-255
	G1 green dimming, from dark to bright

	CH 3
	000-255
	B1 blue dimming, from dark to bright

	...
	...
	...

	...
	...
	...

	CH 142
	000-255
	R48 red dimming, from dark to bright

	CH 143
	000-255
	G48 green dimming, from dark to bright

	CH 144
	000-255
	B48 blue dimming, from dark to bright


154 Channel (point control)
	channel
	value
	function declaration

	CH 1
	000-255
	Total dimming, from dark to bright

	CH 2
	000-255
	Total strobe, from slow to fast

	CH 3
	000-255
	R1 red dimming, from dark to bright

	CH 4
	000-255
	G1 green dimming, from dark to bright

	CH 5
	000-255
	B1 blue dimming, from dark to bright

	...
	...
	...

	...
	...
	...

	CH 144
	000-255
	R48 red dimming, from dark to bright

	CH 145
	000-255
	G48 green dimming, from dark to bright

	CH 146
	000-255
	B48 blue dimming, from dark to bright

	CH 147
	000-255
	W1 white dimming, from dark to bright

	...
	...
	...

	CH 154
	000-255
	W8 white dimming, from dark to bright


Key operation and digital tube display instructions

1. Key description

[image: image1.jpg]585888
© ©® © O

MENU UP DOWN ENTER




2. Digital tube display menu description

	number
	content
	value
	function declaration

	1
	A001
	001-512
	The DMX512 address setting

	2
	8H
	4/6/12/144/154
	Channel mode setting

	3
	R255
	000 - 255
	Red dimming, from dark to bright

	4
	G255
	000 - 255
	Green dimming, from dark to bright

	5
	B255
	000 - 255
	Blue dimming, from dark to bright

	6
	W255
	000 - 255
	White dimming, from dark to bright

	7
	F000
	005 - 255
	Total strobe, from slow to fast

	8
	AU01
	01 - 08
	UP to switch the effect
	Press ENTER to set the speed SP01-SP15

	9
	EF01
	00 - 88
	UP to switch the effect
	Press ENTER to set the speed SP01-SP15

	10
	UF01
	00 - 42
	UP to switch the effect
	Press ENTER to set the speed SP01-SP15

	11
	SU01
	01 - 06
	Sound mode
	Press ENTER to set the sensitivity SE01-SE15


