3. Supporting the remote control button operation instructions

ON/OFF: on/off

Strobe: strobe

DMX:A001 menu

Auto: Self-propelled mode 1

MENU: menu plus

Jump: Self-propelled mode 28

Falsh: self-propelled mode 30

Sound:SU01-voice control

Add numerical value: ↑

Numerical subtraction: ↓

RED: w1

GREEN：w2

BLUE：w3

AMBER：w1w2

CYAN: w2w3

ROSEO: w1w3

WHITE：w1w2w3

9-pcs golden light instructions

- The DMX channel menu

11 Channel (Basic Function)

| channel | value | function declaration |
| --- | --- | --- |
| CH 1 | 000-255 | Total dimming, from dark to bright |
| CH 2 | 005-255 | Total strobe, from slow to fast |
| CH 3 | 000-004 | Perform the (CH 9-11) RGB static dimming |
|  | 005-009 | Static color, CH4 Select (WHITE1,WHITE2,WHITE3) |
|  | 010-014 | Breathing effect, CH5 Select color |
|  | 015-019 | Jump change effect, CH5 Select color |
|  | 020-024 | Gradient change effect, (full color effect) |
|  | 025-029 | Pulse change effect, CH5 Select color |
|  | 030-037 | Autorun effect-1, CH5 Select color(cycle autorun) |
|  | 038-045 | Autorun effect-2, CH5 Select color |
|  | 046-053 | Autorun effect-3, CH5 Select color |
|  | ... | ... |
|  | ... | ... |
|  | 214-221 | Autorun effect-24, CH5 Select color |
|  | 222-229 | Autorun effect-25, (full color effect) |
|  | 230-239 | Autorun effect-26, (full color effect) |
|  | 240-244 | Sound mode 1, CH5 Select color |
|  | 245-249 | Sound mode 2, CH5 Select color |
|  | 250-255 | Sound mode 3, CH5 Select color |
| CH 4 | 000-255 | Static color mode: WHITE1,WHITE2,WHITE3 Autorun effect: adjustment speed, from slow to fast . Sound mode: adjustment sensitivity, from low to high (The highest sensitivity is the sensitivity set by the menu) . |
| CH 5 | 000-028 | Default color |
|  | 029-056 | Red, CH3 Manual color selection |
|  | 057-085 | Green, CH3 Manual color selection |
|  | 086-113 | Blue, CH3 Manual color selection |
|  | 114-142 | Yellow, CH3 Manual color selection |
|  | 143-170 | Purple, CH3 Manual color selection |
|  | 171-199 | Cyan, CH3 Manual color selection |
|  | 200-227 | White, CH3 Manual color selection |
|  | 228-255 | User-defined color(CH6-CH8), CH3 Manual color selection |
| CH 6 | 000-255 | Autorun effect color: WHITE1 dimming, from dark to bright |
| CH 7 | 000-255 | Autorun effect color: WHITE2 dimming, from dark to bright |
| CH 8 | 000-255 | Autorun effect color: WHITE3 dimming, from dark to bright |
| CH 9 | 000-255 | Background color: WHITE1 dimming, from dark to bright |
| CH 10 | 000-255 | Background color: WHITE2 dimming, from dark to bright |
| CH 11 | 000-255 | Background color: WHITE3 dimming, from dark to bright |

3 Channel (whole lamp)

| channel | value | function declaration |
| --- | --- | --- |
| CH 1 | 000-255 | WHITE1 dimming, from dark to bright |
| CH 2 | 000-255 | WHITE2 dimming, from dark to bright |
| CH 3 | 000-255 | WHITE3 dimming, from dark to bright |

5 Channel (whole lamp)

| channel | value | function declaration |
| --- | --- | --- |
| CH1 | 000-255 | Total dimming, from dark to bright |
| CH 2 | 005-255 | Total strobe, from slow to fast |
| CH 3 | 000-255 | WHITE1 dimming, from dark to bright |
| CH 4 | 000-255 | WHITE2 dimming, from dark to bright |
| CH 5 | 000-255 | WHITE3 dimming, from dark to bright |

108 Channel (point control)

| channel | value | function declaration |
| --- | --- | --- |
| CH 1 | 000-255 | WHITE1-1 dimming, from dark to bright |
| CH 2 | 000-255 | WHITE2-1 dimming, from dark to bright |
| CH 3 | 000-255 | WHITE3-1 dimming, from dark to bright |
| ... | ... | ... |
| ... | ... | ... |
| CH 106 | 000-255 | WHITE1-36 dimming, from dark to bright |
| CH 107 | 000-255 | WHITE2-36 dimming, from dark to bright |
| CH 108 | 000-255 | WHITE3-36 dimming, from dark to bright |

110 Channel (point control)

| channel | value | function declaration |
| --- | --- | --- |
| CH1 | 000-255 | Total dimming, from dark to bright |
| CH 2 | 005-255 | Total strobe, from slow to fast |
| CH 3 | 000-255 | WHITE1-1 dimming, from dark to bright |
| CH 4 | 000-255 | WHITE2-1 dimming, from dark to bright |
| CH 5 | 000-255 | WHITE3-1 dimming, from dark to bright |
| ... | ... | ... |
| CH 108 | 000-255 | WHITE1-36 dimming, from dark to bright |
| CH 109 | 000-255 | WHITE2-36 dimming, from dark to bright |
| CH 110 | 000-255 | WHITE3-36 dimming, from dark to bright |

- Key operation and digital tube display instructions

1. Key description

MENU: function key	UP: Key+ DOWN: key- ENTER: Confirm key

2. Digital tube display menu description

| number | content | value | function declaration |  |
| --- | --- | --- | --- | --- |
| 1 | A001 | 001 - 512 | The DMX512 address setting |  |
| 2 | 11CH | 3/5/11/108/110 | Channel mode setting |  |
| 3 | R255 | 000 - 255 | WHITE1 dimming, from dark to bright |  |
| 4 | G255 | 000 - 255 | WHITE2 dimming, from dark to bright |  |
| 5 | B255 | 000 - 255 | WHITE3 dimming, from dark to bright |  |
| 6 | F000 | 005 - 255 | Total strobe, from slow to fast |  |
| 7 | CL01 | 01 - 07 | red, green, blue, yellow, purple, cyan, white |  |
| 8 | EF01 | 01 - 30 | UP to switch the effect | Press ENTER to set the speed SP01-SP15 |
| 9 | SU01 | 01 - 03 | Sound mode | Press ENTER to set the sensitivity SE01-SE15 |