Double row of 12 UV purple light lights

operating instruction

MENU  UP   DOWN  ENTER

Menu (switch) Increase and decrease confirmation (come in)

MENU  UP   DOWN  ENTER

Menu (switch) Increase and decrease confirmation (come in)

Thank you very much for purchasing our company's double row of 12 UV purple light lights. For your personal safety and better use of this product, please read this manual carefully before using it, and operate according to the procedures, so as to avoid personal safety damage and lamp damage caused by misuse.

● packing list

Check the following when you open the lamp box:

Double row 12 UV light light....................... 1

Double row 12 UV purple light instruction manual ·...................... 1 copy

Iron stand ·........................ 2

Accessories Knob Screws and Washers......................... 2 Groups

Purple light remote control (remote control version belt).......................... 1

● technical parameter:

Power supply: AC 90~240V, 50H z / 60H z

Rated power,: 40W

Case material: aluminum profile

Light source: 243W LED lamp beads

Glow-light color: purple light

Wavelength: 395-405nm

Optical flux: 1.2-2.0M

Light source life: more than 3,000 hours

Suitable for use: bar, di bar, ballroom, KTV private room, family Party, etc

Power connection: power cable

Signal connection: signal input line / output line

Control channel: 7 DMX-512 signal channels

Control mode: DMX-512 signal control, sound control, self-walk, breathing, strobe and primary-slave mode

Cooling system: the intelligent fan controls the heat dissipation, and the fan does not turn when the product is energized but not lit

Working environment: indoor, packaging mode: carton

Inner box size: 62 * 8 * 8CM

Outer box size: 63.5*42*34.5CM

Product net weight: 1.25KG

Product Gross weight: 1.45KG

Box gross weight: 30.05KG (20pcs / box)

● direction for use:

After opening the packing box, carefully check whether the lamps are damaged by transportation, such as whether the screws are loose, and whether the lenses are broken. Please check before the power-on. Check whether the lamp placement position is correct and stable, and check whether the power supply voltage is the same as that of the product. This equipment belongs to level 1 protection equipment, so the ground wire must be grounded and completed by professionals. Check whether the power supply voltage is normal before making use.

● Display Features:

This product adopts display mode and DMX mode operation, specific settings as follows:

| order number | Display mode | function declaration |
| --- | --- | --- |
| 1 | d 001-d512 | DMX console mode, add and subtract address code values through the upper and lower keys |
| 2 | A 1.00-A 1.20 | Forward horse racing mode, the larger the last two display values, the faster the jump speed |
| 3 | A 2.00-A 2.20 | In reverse racing mode, the larger the last two displays, the faster the speed |
| 4 | A 3.00-A 3.20 | Respiratory mode, the latter two bits show larger values, and faster breathing speed |
| 5 | A 4.00-A 4.20 | Full effect cycle demonstration, the larger the last two display value, the faster the demonstration speed |
| 6 | A 5.00-A 5.20 | Voice control mode, sound when strobe, silent when closed light, the greater the last two display value, the higher the sensitivity of sound control |
| 7 | ALon | Full bright mode |
| 8 | FF.00-FF.20 | In strobe mode, the larger the last two display values, the faster the strobe speed |
| Note: Long press the "Enter" key to enter the brightness adjustment interface, "rL01-rL99", the larger the value, the higher the brightness of the lamp bead |  |  |

● DMX Channel Features:

The product adopts standard DMX-512 signal with 7 control channels, and the specific control contents are as follows:

| number of channels | The channel value | Functional detailed explanation |
| --- | --- | --- |
| 1CH | 0-255 | Total dimming (combined with the CH2-CH6 channel) |
| 2CH | 0-255 | First route light adjusts brightness (from dark to bright) |
| 3CH | 0-255 | Second route light adjusts brightness (from dark to bright) |
| 4CH | 0-255 | Third-way light adjustment brightness (from dark to bright) |
| 5CH | 0-255 | Fourth route light adjustment brightness (from dark to bright) |
| 6CH | 0-255 | 0-5: No strobe; 6-255: The strobe (from slow to fast), the larger the value, the faster |
| 7CH | 0-50 | Invalid channel value domain |
|  | 51-90 | Is running a horseMode, the larger the value in this frequency band, the speed is changed from fast to slow |
|  | 91-130 | Reverse horse racingEffect, the greater the value in this frequency band, the speed from fast to slow |
|  | 131-170 | Respiratory mode, the greater the value in this frequency band, the speed is changed from fast to slow |
|  | 171-210 | Full effect presentation. The greater the value in this frequency band, the speed is changed from fast to slow |
|  | 211-255 | Sound Control Mode,Sound when strobe, silent when closed light |

● Maintenance and Maintenance:

- Do not keep your eyes against the semiconductor light body for a long time, so as not to harm the eyes;

- Try to eliminate dust, dirt and smoke and oil artificially laid or even into the lamp body, the use of the environment as far as possible to keep the product clean;

- Please use the professional glass cleaning agent regularly every month and clean the PVC plate with a velvet cloth to ensure that the maximum brightness output and can prolong the service life of the light source.