LED Bee Eye 6-in-1

Product instructions for use

AC Voltage: AC100-240V, 50 / 60Hz

Total power: 80W

Feng eye: 6 RGBW four-in-one 8W lamp beads

Pattern: 3 pictures

Red and green laser: 1 red laser 650nm 1 green laser 532nm

Green light: 1 red laser 650nm 1 green laser 532nm

Strobe: 2 yellow light

Working mode: DMX512 signal / main-slave / self-walking / voice control / remote control

Remote control: wireless 433 MHZ

- Function Settings

| menu | numeric value | description |
| --- | --- | --- |
| A1 | 1~9 | Beacon's eyes are red and bright, and the motor speed is moving from slow to fast |
| A2 |  | The beacon eyes red green bright, the motor speed from slow to fast |
| A3 |  | Feng eyes red blue bright, the motor speed from slow to fast |
| A4 |  | The beacon is red and white bright, the motor speed from slow to fast |
| A5 |  | Pattern bright, the motor speed from slow to fast |
| A6 |  | Red and green laser bright, the motor speed from slow to fast |
| A7 |  | Strobe, strobe speed from slow to fast |
| A8 |  | The sky star red and green laser switch, the speed from slow to fast |
| C1 |  | Combination from the walk 1, the speed from slow to fast |
| C2 |  | Combination from the walk 2, the speed from slow to fast |
| S1 | 1~9 | Voice control 1, voice control sensitivity increases with the value |
| S2 |  | Voice control 2, voice control sensitivity increases with the increase of the value |
| S2 |  | Voice control 3, voice control sensitivity increases with the increase of the value, silent control light |
| D | 001~512 | DMX or from the machine |

MENU is the menu switch key, UP, DOWN is the value switch key, and ENTER is the confirmation key.

- DMX control

| channel | The channel value | functional description |
| --- | --- | --- |
| CH1 | 0~255 | Total dimming |
| CH2 | 0~255 | Beacon red light dimming |
| CH3 | 0~255 | Beacon green light dimming |
| CH4 | 0~255 | Beacon blue light dimming |
| CH5 | 0~255 | Beacon eyes white light dimming |
| CH6 | 0~4 | There is no frequency flash in the eye |
|  | 5~255 | There is no stroboscopic strobe, the larger the channel value, the faster the strobe speed |
| CH7 | 0~4 | Beacon motor stopped |
|  | 5~255 | The beacon eye motor rotates clockwise, and the larger the channel value, the faster the speed |
|  | 5~255 | The beacon eye motor rotates counterclockwise, and the larger the channel value is, the slower the speed is |
| CH8 | 0~255 | Pattern dimming |
| CH9 | 0~9 | The pattern motor stops |
|  | 10~255 | The pattern motor rotates, and the larger the channel value, the faster the speed |
| CH10 | 0~4 | The red laser is not bright |
|  | 5~255 | Red laser bright |
| CH11 | 0~4 | Green laser is not bright |
|  | 5~255 | Green laser bright |
| CH12 | 0~9 | The laser motor stops |
|  | 5~255 | The laser motor rotates, and the larger the channel value, the faster the speed |
| CH13 | 0~4 | The sky star laser is not bright |
|  | 5~127 | The sky star laser is red and bright |
|  | 128~191 | The sky star laser is green and bright |
|  | 192~255 | The sky star laser is red and green |
| CH14 | 0~49 | CH1~CH13 valid |
|  | 0~49 | macro |

Instructions for using the remote control

| key | function |
| --- | --- |
| O N | opening the light |
| DMX | Console 512 mode |
| OFF | Closed light |
| SPEED- | Speed reduction |
| AUTO | Self-moving mode switch |
| SPEED+ | Speed and |
| MIC+ | Sound control sensitivity plus |
| VOICE | Sound control mode switch |
| MIC- | Sound control sensitivity decreased |
| 1 | A1 pattern |
| 2 | A2 pattern |
| 3 | A3 pattern |
| 4 | A4 pattern |
| 5 | A5 pattern |
| 6 | A6 pattern |
| 7 | A7 pattern |
| 8 | A8 pattern |
| 9 | C1 pattern |