Technical parameters:

Voltage: AC110-220V, 50-60HZ

Power: 60W

Light source: 6 pieces of 500mw(638)red laser, 6 pieces of 5050 dye

Channel: 8/12/32CH

Control mode: Sound control, master-slave, self-propelled, DMX
Operation Manual of LED Stage Lighting Equipment
6 Laser+6 Dyeb
Please read the user manual carefully before use
Function description                                                   
	Addr
	001-512
	512 Addr Setting

	Chnd
	8/15/35CH
	Channel Settings

	Slnd       
	Auto
	Auto

	
	Soun
	Sound

	
	Slve 1
	Console mode

	
	Slve 2
	Auxiliary mode

	Shnd
	LF 0-1
	0 mode LA/LE operation is valid, 1 mode LA/LE operation is invalid, and the combination effect is adopted

	
	LA 0-9
	laser

	
	LE 0-9
	dyeb

	Auto  Speed
	0-100
	Auto speed adjustment

	Sound  Sense
	0-100
	Sound sensitivity adjustment

	LED
	OFF
	Screen off

	
	ON
	Screen on

	Disp
	Yes
	Turn off inverted display

	
	ON
	Turn on reverse display

	RPan  
	OFF
	X motor forward rotation

	
	ON
	X motor reverses

	Rest
	YES
	Reset


Channel table

	8CH
	12CH
	32CH
	function
	explain

	1
	1
	1
	0-255
	Y motor running

	2
	2
	2
	0-255
	Y motor speed

	3
	3
	3
	0-255
	Total dimming

	4
	4
	4
	0-255
	Stroboscopic speed regulation

	
	5
	
	0-255
	Laser 1-6 is on and off

	
	6
	
	0-255
	Dyeb 1-6 red on/off

	
	7
	
	0-255
	Dyeb 1-6 green on/off

	
	8
	
	0-255
	Dyeb 1-6 blue on/off

	
	
	5
	0-255
	Laser 1 on /off

	
	
	6
	0-255
	Laser 2 on /off

	
	
	7
	0-255
	Laser 3 on /off

	
	
	8
	0-255
	Laser 4 on /off

	
	
	9
	0-255
	Laser 5 on /off

	
	
	10
	0-255
	Laser 6 on /off

	
	
	11
	0-255
	Dyeb 1 red on/off

	
	
	12
	0-255
	Dyeb 1 green on/off

	
	
	13
	0-255
	Dyeb 1 blue on/off

	
	
	...
	...
	...

	
	
	26
	0-255
	Dyeb 6 red on/off

	
	
	27
	0-255
	Dyeb 6 green on/off

	
	
	28
	0-255
	Dyeb 6 blue on/off


Channel table

	5
	9
	29
	0-21
	No effect

	
	
	
	22-138
	Laser auto walking built-in effect (1-9)

	
	
	
	139-255
	laser sound control built-in effect（1-9）

	6
	10
	30
	0-21
	No effect

	
	
	
	22-138
	dyeb auto walking built-in effect (1-9)

	
	
	
	139-255
	dyeb sound control built-in effect（1-9）

	7
	11
	31
	0-255
	Speed/sound sensitivity

	8
	12
	32
	250-255
	Reset


