Three Head RGB Animation Laser Light

User’s Manual

MENU

UP

ENTER

DOWN

Thank you very much for purchasing the three head RGB Animation laser light. For your personal safety and better use of this product, please read this manual carefully before use and follow the instructions to avoid personal injury and damage to the lamp.

● Packing List：

Please check if you have the following items when you open the light box:

Three head animation laser light             ··········· one

Power cord                                ··········· one

Manual                                   ··········· one

● Technical parameters：

Power supply：AC100～240V ，50Hz/60Hz

Rated power：120W

Laser power：1800mw/3000mw

Luminous colors：RGB

Chassis material：aluminum+iron

Scanning system: 256 + 11 laser effects + galvanometer scanning system 15kpps / ± 25 °

Power connection: power cord

Signal connection: signal input line / output line

Control channel: 6/34 DMX-512 signal channels

Control mode: DMX-512 signal control, voice control, auto and master-slave mode

Cooling system: Fan forced cooling system

Working environment: indoor

Packing: carton

Product size：50.3*17.5*7.5cm（without handle）

Inner box size： CM

Carton size： CM

N.W：

G.W： KG

● Instructions for use:

After opening the packing box, carefully check whether the lamp is damaged due to transportation, such as whether the screw is loose, whether the lens is broken, etc. Please check before powering on. Check whether the lamp is placed in the correct and stable position, and check whether the power supply voltage is the same as that of this product. This equipment is a class 1 protection device, so the ground wire of the power supply must be grounded and completed by a professional. Check whether the power supply voltage is normal before use.

● Display Function：This product uses display mode operation mode and DMX mode operation mode, the specific settings are as follows：

| NO. | Display | Function Instruction |
| --- | --- | --- |
| 1 | A001-A512 | Console Mode 1 (Professional Mode) |
| 2 | E001-E512 | Console Mode 2 (Simple Mode) |
| 3 | SE 1 | NO.1 Gallery, self-propelled mode, press the "Enter" key to enter the voice control mode ("▪" appears on the display) |
| 4 | SE 2 | NO.2 Gallery, self-propelled mode, press the "Enter" key to enter the voice control mode ("▪" appears on the display) |
| 5 | SE 3 | NO.3 Gallery, self-propelled mode, press the "Enter" key to enter the voice control mode ("▪" appears on the display) |

● DMX Channel Function：This product adopts standard DMX-512 signal, with a total of 6/34 control channels. The specific control contents are as follows:

| Channel | 6CH Channel mode（E001） | 34CH Channel mode（A001） |
| --- | --- | --- |
| CH1 | 0：Turn off the lights； 1-255 Auto mode | 0：Turn off the lights；1-99：auto；100-199：sound control； 200-254：keep；255：Pattern A turns off the light |
| CH2 | 0-127: Self-propelled selection of dynamic gallery No. 1; 128-255: No. 2 dynamic gallery voice control mode | Out-of-bounds mode and pattern size (the larger the value in the segment, the smaller the pattern): 0-49: out-of-bounds crossing; 50-99: out-of-bounds retracement 100-149: Out-of-boundary blanking; 150-199: Out-of-boundary blanking (the larger the value in the segment, the larger the pattern); 200-255: reserved |
| CH3 | 240-255: No. 3 dynamic gallery self-propelled | Gallery selection: 0-239: Gallery 1; 240-255: Gallery 0 |
| CH4 | 1-255: No. 4 dynamic pattern group library selection | Pattern selection: each value plays a pattern, when the value is greater than the number of patterns, it is the last pattern |
| CH5 | 0-31: color pattern; 32-255 pattern 7 kinds of monochrome options | Pattern zoom (the greater the value in the segment, the faster the speed): 0-127: static size; 128-159: dynamic zoom 160-191: dynamic reduction; 192-223: dynamic zoom 224-255: Dynamic flip zoom |
| CH6 | 1-255 speed adjustment (from slow to fast) | Pattern rotation (the greater the value in the segment, the faster the speed): 0-127: static rotation; 128-159: dynamic 2 weeks forward and reverse 160-191: dynamic 1 week positive and negative reversal; 192-223: dynamic forward turn; 224-255: dynamic reverse turn |
| CH7 |  | Horizontal movement (the greater the value in the segment, the faster the speed): 0-127: Static translation; 128-159: Dynamically push up the wave 160-191: Dynamically push down the wave; 192-223: Dynamically move left; 224-255: Dynamically move right |
| CH8 |  | Vertical movement (the greater the value in the segment, the faster the speed): 0-127: static vertical shift; 128-159: dynamic right push distortion 160-191: dynamic left push distortion; 192-223: dynamic downward movement; 224-255: dynamic upward movement |
| CH9 |  | Y axis flip (the greater the value in the segment, the faster the speed): 0-127: static flip; 128-159: dynamic push-up distortion 160-191: Dynamic push-down distortion; 192-223: Dynamic positive flip; 224-255: Dynamic reverse flip |
| CH10 |  | X axis flip (the greater the value in the segment, the faster the speed): 0-127: static flip; 128-159: dynamic right push distortion 160-191: Dynamic left push distortion; 192-223: Dynamic positive flip; 224-255: Dynamic reverse flip |
| CH11 |  | Forced color setting: 0: primary color; 1-255: change color every n points |
| CH12 |  | Color change (the greater the value in the segment, the faster the speed): 0-7: primary color / 8-15: red / 16-23: yellow / 24-31: green / 32-39: indigo / 40-47: blue / 48-55: purple / 65-63: white 64-95: full-color red, green and blue discoloration 96-127: full picture yellow indigo change color; 128-159: full picture seven color change; 160-191: colorful change color 192-223: roaming colors; 224-255: reverse roaming colors |
| CH13 |  | Node brightness: 0-63: normal blanking; 64-127: display broken pen 128-159: display retrace line; 224-255: reserved |
| CH14 |  | Node expansion: 0-255: expansion points (when CH15 <= 127) 0-255: Delay after full expansion (when CH15> = 128) |
| CH15 |  | Gradual drawing (the greater the value in the segment, the faster the speed): 0-63: Forward manual deployment (the amount of deployment is determined by CH14) 64-127: reverse manual deployment (the amount of deployment is determined by CH14) 128-159: Dynamic Gradient Effect A; 160-191: Dynamic Gradient Effect B; 192-223: Dynamic Gradient Effect C 224-255: Dynamic gradual drawing effect D |
| CH16 |  | Distortion: 0-255: adjust the distortion of various distortion effects, the larger the value, the smaller the distortion |
| CH17 |  | Filter selection (the larger the value in the segment, the smaller the frame): 0-19: filter 1 20-39: filter 2 / 40-59: filter 3 ... 240-255: filter 13 |
| CH18 |  | 0: turn off the light; 1-99: self-propelled; 100-199: voice control; 200-254: reserved; 255: pattern B off |
| CH19 |  | Out-of-bounds mode and pattern B size (the larger the value in the segment, the smaller the pattern): 0-49: out-of-bounds crossing; 50-99: out-of-bounds retracement 100-149: Out-of-boundary blanking; 150-199: The greater the value in the out-of-boundary blanking section, the larger the pattern); 200-255: Reserved |
| CH20 |  | (No function) reserved |
| CH21 |  | B pattern selection: each value plays a pattern, when the value is greater than the number of patterns, it is the last pattern |
| CH22 |  | B pattern zoom (the greater the value in the segment, the faster the speed): 0-127: static size; 128-159: dynamic zoom 160-191: dynamic reduction; 192-223: dynamic zoom 224-255: Dynamic flip zoom |
| CH23 |  | B pattern rotation (the greater the value in the segment, the faster the speed): 0-127: static rotation; 128-159: dynamic 2 weeks forward and reverse 160-191: dynamic 1 week positive and negative reversal; 192-223: dynamic forward turn; 224-255: dynamic reverse turn |
| CH24 |  | Horizontal movement (the greater the value in the segment, the faster the speed): 0-127: Static translation; 128-159: Dynamically push up the wave 160-191: Dynamically push down the wave; 192-223: Dynamically move left; 224-255: Dynamically move right |
| CH25 |  | Vertical movement (the greater the value in the segment, the faster the speed): 0-127: static vertical shift; 128-159: dynamic right push distortion 160-191: dynamic left push distortion; 192-223: dynamic downward movement; 224-255: dynamic upward movement |
| CH26 |  | Y axis flip (the greater the value in the segment, the faster the speed): 0-127: static flip; 128-159: dynamic push-up distortion 160-191: Dynamic push-down distortion; 192-223: Dynamic positive flip; 224-255: Dynamic reverse flip |
| CH27 |  | X axis flip (the greater the value in the segment, the faster the speed): 0-127: static flip; 128-159: dynamic right push distortion 160-191: Dynamic left push distortion; 192-223: Dynamic positive flip; 224-255: Dynamic reverse flip |
| CH28 |  | Forced color setting: 0: primary color; 1-255: change color every n points |
| CH29 |  | Color change (the greater the value in the segment, the faster the speed): 0-7: primary color / 8-15: red / 16-23: yellow / 24-31: green / 32-39: indigo / 40-47: blue / 48-55: purple / 65-63: white 64-95: full-color red, green and blue discoloration 96-127: full picture yellow indigo change color; 128-159: full picture seven color change; 160-191: colorful change color 192-223: roaming colors; 224-255: reverse roaming colors |
| CH30 |  | Node brightness: 0-63: normal blanking; 64-127: display broken pen 128-159: display retrace line; 224-255: reserved |
| CH31 |  | Node expansion: 0-255: expansion points (when CH32 <= 127) 0-255: Delay after full expansion (when CH32> = 128) |
| CH32 |  | Gradual drawing (the greater the value in the segment, the faster the speed): 0-63: Forward manual deployment (the amount of deployment is determined by CH14) 64-127: reverse manual deployment (the amount of deployment is determined by CH14) 128-159: Dynamic gradual drawing effect A; 160-191: Dynamic gradual drawing effect B192-223: Dynamic gradual drawing effect C 224-255: Dynamic gradual drawing effect D |
| CH33 |  | Distortion: 0-255: adjust the distortion of various distortion effects, the larger the value, the smaller the distortion |
| CH34 |  | Filter selection (the larger the value in the segment, the smaller the frame): 0-19: frame size 20-255: reserved |

● Maintenance:

1. Try to prevent the artificial laying of dust, dirt and smoke oil even into the lamp body, use the environment to keep the laser performer as clean as possible;

2. Please use professional glass cleaner to clean the lens regularly every month to ensure the maximum laser output and extend the life of the light source.

3. Please do not keep your eyes on the semiconductor luminous body for a long time, so as not to damage your eyes.