Mini-Storm

一：Operation manu

| Addr | A001-A512 | Addr A001-A512 |  |
| --- | --- | --- | --- |
| Chnd | CH23 | CH101 | Channel selection |
| Slnd | Mast | Master |  |
|  | d512 | Slave |  |
|  | FLS | Fast lighting show |  |
| SHOU | ON | Hold | Sound lost |
|  | Off | Back out | Sound lost |
| Soun | Su | 0--20 | Sound action |
|  | Se | 0--99 | Sound sensitivity |
| Auto | Au | 0--20 | Auto action |
|  | Sp | 0-9 | Auto |
| RF | ON | RF433-On | Master |
|  | OFF | RF433-Off |  |
| dISP | dISP | Display reverse |  |
| Ver | V10 | Version |  |
| Load | NO | No |  |
|  | YES | Reset | Factory reset |

2. Channel

| 23-ch | Function | Value | Description |
| --- | --- | --- | --- |
| 1 | Beam-Total dimmer | 000--255 | Beam-Total dimmer 0%~ 100% |
| 2 | Beam-Strobe | 000--255 | 0%~ 100% |
| 3 | 5050 White-Total dimmer | 000--255 | 0%~ 100% |
| 4 | 5050 White- Strobe | 000--255 | 0%~ 100% |
| 5 | 5050RGB- Total dimmer | 000--255 | 0%~ 100% |
| 6 | 5050RGB- Strobe | 000--255 | 0%~ 100% |
| 7 | Beam-dimmer | 000--255 | Red 0%~ 100% |
| 8 | Beam-dimmer | 000--255 | Green 0%~ 100% |
| 9 | Beam-dimmer | 000--255 | Blue 0%~ 100% |
| 10 | Beam-dimmer | 000--255 | White 0%~ 100% |
| 11 | 5050 White-dimmer | 000--255 | 0%~ 100% |
| 12 | 5050RGB-Red dimmer | 000--255 | 0%~ 100% |
| 13 | 5050RGB-Green dimmer | 000--255 | 0%~ 100% |
| 14 | 5050RGB-Green dimmer | 000--255 | 0%~ 100% |
| 15 | Laser-Red | 000--007 | No function |
|  | Laser-Red | 008--255 | On |
| 16 | Laser-Green | 000--007 | No function |
|  | Laser-Green | 008--255 | On |
| 17 | Laser | 000--009 | No function |
|  | Laser Motor | 010--134 | Rotate counterclockwise from fast to slow |
|  | Laser Motor | 135--245 | Rotate clockwise from slow to fast |
|  | Laser | 246--255 | Stop turning |
| 18 | 5050RGB | 000-007 | No function |
|  | 5050RGB | 008--247 | Program (Annex 1 for details) |
|  | 5050RGB | 248--255 | Sound control |
| 19 | 5050RGB | 000--255 | Program speed 0%~ 100% |
| 20 | 5050White | 000-007 | No function |
|  | 5050White | 008--247 | Program (Annex 1 for details) |
|  | 5050White | 248--255 | Sound control |
| 21 | 5050White | 000--255 | Program speed 0%~ 100% |
| 22 | Beam | 000-007 | No function |
|  | Beam | 008--247 | Program (Annex 1 for details) |
|  | Beam-Sound control | 248--255 |  |
| 23 | Beam | 000--255 | Program speed 0%~ 100% |

| 101-ch | Function | Value | Description |
| --- | --- | --- | --- |
| 1 | Beam-Total dimmer | 000--255 | 0%~ 100% |
| 2 | Beam-Strobe | 000--255 | 0%~ 100% |
| 3 | Beam1-Red dimmer | 000--255 | 0%~ 100% |
| 4 | Beam1-Green dimmer | 000--255 | 0%~ 100% |
| 5 | Beam1-Green dimmer | 000--255 | 0%~ 100% |
| 6 | Beam1-White dimmer | 000--255 | 0%~ 100% |
| 7 | Beam2-Red dimmer | 000--255 | 0%~ 100% |
| 8 | Beam2-Green dimmer | 000--255 | 0%~ 100% |
| 9 | Beam2-Green dimmer | 000--255 | 0%~ 100% |
| 10 | Beam2-White dimmer | 000--255 | 0%~ 100% |
| 11 | 5050White-Total dimmer | 000--255 | 0%~ 100% |
| 12 | 5050White-Strobe | 000--255 | 0%~ 100% |
| 13 | 5050White1-Dimmer | 000--255 | 0%~ 100% |
| 14 | 5050White2-Dimmer | 000--255 | 0%~ 100% |
| 15 | 5050White3-Dimmer | 000--255 | 0%~ 100% |
| 16 | 5050White4-Dimmer | 000--255 | 0%~ 100% |
| 17 | 5050White5-Dimmer | 000--255 | 0%~ 100% |
| 18 | 5050White6-Dimmer | 000--255 | 0%~ 100% |
| 19 | 5050 RGB-Total dimmer | 000--255 | 0%~ 100% |
| 20 | 5050 RGB-Strobe | 000--255 | 0%~ 100% |
| 21 | 5050RGB-1-Red dimmer | 000--255 | 0%~ 100% |
| 22 | 5050RGB-1-Green dimmer | 000--255 | 0%~ 100% |
| 23 | 5050RGB-1-Green dimmer | 000--255 | 0%~ 100% |
| 24 | 5050RGB-2-Red dimmer | 000--255 | 0%~ 100% |
| 25 | 5050RGB-2-Green dimmer | 000--255 | 0%~ 100% |
| 26 | 5050RGB-2-Green dimmer | 000--255 | 0%~ 100% |
| 27 | 5050RGB-3-Red dimmer | 000--255 | 0%~ 100% |
| 28 | 5050RGB-3-Green dimmer | 000--255 | 0%~ 100% |
| 29 | 5050RGB-3-Green dimmer | 000--255 | 0%~ 100% |
| 30 | 5050RGB-4-Red dimmer | 000--255 | 0%~ 100% |
| 31 | 5050RGB-4-Green dimmer | 000--255 | 0%~ 100% |
| 32 | 5050RGB-4-Green dimmer | 000--255 | 0%~ 100% |
| 33 | 5050RGB-5-Red dimmer | 000--255 | 0%~ 100% |
| 34 | 5050RGB-5-Green dimmer | 000--255 | 0%~ 100% |
| 35 | 5050RGB-5-Green dimmer | 000--255 | 0%~ 100% |
| 36 | 5050RGB-6-Red dimmer | 000--255 | 0%~ 100% |
| 37 | 5050RGB-6-Green dimmer | 000--255 | 0%~ 100% |
| 38 | 5050RGB-6-Green dimmer | 000--255 | 0%~ 100% |
| 39 | 5050RGB-7-Red dimmer | 000--255 | 0%~ 100% |
| 40 | 5050RGB-7-Green dimmer | 000--255 | 0%~ 100% |
| 41 | 5050RGB-7-Green dimmer | 000--255 | 0%~ 100% |
| 42 | 5050RGB-8-Red dimmer | 000--255 | 0%~ 100% |
| 43 | 5050RGB-8-Green dimmer | 000--255 | 0%~ 100% |
| 44 | 5050RGB-8-Green dimmer | 000--255 | 0%~ 100% |
| 45 | 5050RGB-9-Red dimmer | 000--255 | 0%~ 100% |
| 46 | 5050RGB-9-Green dimmer | 000--255 | 0%~ 100% |
| 47 | 5050RGB-9-Green dimmer | 000--255 | 0%~ 100% |
| 48 | 5050RGB-10-Red dimmer | 000--255 | 0%~ 100% |
| 49 | 5050RGB-10-Green dimmer | 000--255 | 0%~ 100% |
| 50 | 5050RGB-10-Green dimmer | 000--255 | 0%~ 100% |
| 51 | 5050RGB-11-Red dimmer | 000--255 | 0%~ 100% |
| 52 | 5050RGB-11-Green dimmer | 000--255 | 0%~ 100% |
| 53 | 5050RGB-11-Green dimmer | 000--255 | 0%~ 100% |
| 54 | 5050RGB-12-Red dimmer | 000--255 | 0%~ 100% |
| 55 | 5050RGB-12-Green dimmer | 000--255 | 0%~ 100% |
| 56 | 5050RGB-12-Green dimmer | 000--255 | 0%~ 100% |
| 57 | 5050RGB-13-Red dimmer | 000--255 | 0%~ 100% |
| 58 | 5050RGB-13-Green dimmer | 000--255 | 0%~ 100% |
| 59 | 5050RGB-13-Green dimmer | 000--255 | 0%~ 100% |
| 60 | 5050RGB-14-Red dimmer | 000--255 | 0%~ 100% |
| 61 | 5050RGB-14-Green dimmer | 000--255 | 0%~ 100% |
| 62 | 5050RGB-14-Green dimmer | 000--255 | 0%~ 100% |
| 63 | 5050RGB-15-Red dimmer | 000--255 | 0%~ 100% |
| 64 | 5050RGB-15-Green dimmer | 000--255 | 0%~ 100% |
| 65 | 5050RGB-15-Green dimmer | 000--255 | 0%~ 100% |
| 66 | 5050RGB-16-Red dimmer | 000--255 | 0%~ 100% |
| 67 | 5050RGB-16-Green dimmer | 000--255 | 0%~ 100% |
| 68 | 5050RGB-16-Green dimmer | 000--255 | 0%~ 100% |
| 69 | 5050RGB-17-Red dimmer | 000--255 | 0%~ 100% |
| 70 | 5050RGB-17-Green dimmer | 000--255 | 0%~ 100% |
| 71 | 5050RGB-17-Green dimmer | 000--255 | 0%~ 100% |
| 72 | 5050RGB-18-Red dimmer | 000--255 | 0%~ 100% |
| 73 | 5050RGB-18-Green dimmer | 000--255 | 0%~ 100% |
| 74 | 5050RGB-18-Green dimmer | 000--255 | 0%~ 100% |
| 75 | 5050RGB-19-Red dimmer | 000--255 | 0%~ 100% |
| 76 | 5050RGB-19-Green dimmer | 000--255 | 0%~ 100% |
| 77 | 5050RGB-19-Green dimmer | 000--255 | 0%~ 100% |
| 78 | 5050RGB-20-Red dimmer | 000--255 | 0%~ 100% |
| 79 | 5050RGB-20-Green dimmer | 000--255 | 0%~ 100% |
| 80 | 5050RGB-20-Green dimmer | 000--255 | 0%~ 100% |
| 81 | 5050RGB-21-Red dimmer | 000--255 | 0%~ 100% |
| 82 | 5050RGB-21-Green dimmer | 000--255 | 0%~ 100% |
| 83 | 5050RGB-21-Green dimmer | 000--255 | 0%~ 100% |
| 84 | 5050RGB-22-Red dimmer | 000--255 | 0%~ 100% |
| 85 | 5050RGB-22-Green dimmer | 000--255 | 0%~ 100% |
| 86 | 5050RGB-22-Green dimmer | 000--255 | 0%~ 100% |
| 87 | 5050RGB-23-Red dimmer | 000--255 | 0%~ 100% |
| 88 | 5050RGB-23-Green dimmer | 000--255 | 0%~ 100% |
| 89 | 5050RGB-23-Green dimmer | 000--255 | 0%~ 100% |
| 90 | 5050RGB-24-Red dimmer | 000--255 | 0%~ 100% |
| 91 | 5050RGB-24-Green dimmer | 000--255 | 0%~ 100% |
| 92 | 5050RGB-24-Green dimmer | 000--255 | 0%~ 100% |
| 93 | Laser-Red | 000--007 | No function |
|  | Laser-Red | 008--255 | On |
| 94 | Laser-Green | 000--007 | No function |
|  | Laser-Green | 008--255 | On |
| 95 | Laser | 000--009 | No function |
|  | Laser motor | 010--134 | Rotate counterclockwise from fast to slow |
|  | Laser motor | 135--245 | Rotate clockwise from slow to fast |
|  | Laser | 246--255 | Stop turning |
| 96 | 5050RGB | 000-007 | No function |
|  |  | 008--247 | Program (Annex 1 for details) |
|  |  | 248--255 | Sound control |
| 97 | 5050RGB | 000--255 | Program speed 0%~ 100% |
| 98 | 5050White | 000-007 | No function |
|  |  | 008--247 | Program (Annex 1 for details) |
|  |  | 248--255 | Sound control |
| 99 | 5050White | 000--255 | Program speed 0%~ 100% |
| 100 | Beam | 000-007 | No function |
|  |  | 008--247 | Program (Annex 1 for details) |
|  |  | 248--255 | Sound control |
| 101 | Beam | 000--255 | Program speed 0%~ 100% |

Channel Value specification（Annex I）

| Program value |  |
| --- | --- |
| 000--007 | No program |
| 008--019 | Pro1 |
| 020--031 | Pro2 |
| 032--043 | Pro3 |
| 044--055 | Pro4 |
| 056--067 | Pro5 |
| 068--079 | Pro6 |
| 080--091 | Pro7 |
| 092--103 | Pro8 |
| 104--115 | Pro9 |
| 116--127 | Pro10 |
| 128--139 | Pro11 |
| 140--151 | Pro12 |
| 152--163 | Pro13 |
| 164--175 | Pro14 |
| 176--187 | Pro15 |
| 188--199 | Pro16 |
| 200--211 | Pro17 |
| 212--223 | Pro18 |
| 224--235 | Pro19 |
| 236--247 | Pro20 |
| 248--255 | Sound control |

RF433 Remote Control Instructions：

1：Effective distance 20—50M

2：RF433 the remote control is only valid when the light is in master mode

3: This description is only valid for this light

| Remote Control Instructions |  |  |  |
| --- | --- | --- | --- |
| ON/OFF | UP | DOWN | Total dimmer (Up/down) |
| R Red (0—255 up/down) | G Green (0—255 up/down) | B Blue (0—255 up/down) | Blinder |
| W White (0—255up/down) | A Amber (N/A) | P purple (N/A) | Se/Sp Sound sensitivity & Auto speed (up/down) (Valid only Sound&Auto mode) |
| 1 | 2 | 3 | Sound (up/down) |
| 4 | 5 | 6 | Auto (up/down) |
| Combo (Cbo1 Random) (Cbo2 Beam only) (Cbo3 5050RGB only) (Cbo4 Beam+5050RGB) (Cbo5 5050White only) (Cbo6 Beam +5050White) (Cbo7 5050RGB+5050White) (Cbo8 Beam+5050RGB+5050White) (Cbo9 Laser only) (Cbo10Beam+Laser) (Cbo11Laser+5050RGB) (Cbo12Beam+Laser+5050RGB) (Cbo13 Laser+5050White) (Cbo14 Beam+Laser+5050White) (Cbo15 Laser+5050RGB+5050White) (Cbo16 The combination effect is fully illuminated) | Preset Colour select (up/down) | Strobe (up/down) | Fade (up/down) |
| X (N/A) | Y (N/A) | C/G (N/A) | P/F (N/A) |

Technical Parameters:

Input voltage: AC110~240V

Power supply: 150W

Total power: 122

Light source: 2X38pcs 1W high-brightness LED patent LEDs, 8-segment control.

192pcs 5050RGB highlight, 24-segment control.

22pcs 5050White, 6-segment control.

One 150MARedLaser, one 370MAGreenLaser.

Beam angle: 3°

Strobe：1~30Hz

Channel: 23CH &101CH

Control mode: DMX, Sound control, auto-propelled, RF433, wireless DMX (optional)

Dimensions/Weight: 433*177*183mm/ 2.65kg

IP: IP20

Appearance: Metallic

1、 USB:wireless

2、 Panel operation buttons: MENU, UP, Down, Enter

3、 LED display

4、 POWER OUT: Power output

5、 POWER IN: Power input

6、 DMX IN: 3-pin DMX signal input

7、 DMX OUT: 3-pin DMX signal output